You will need a measurement device in inches.

The FRT is an indicator of limits of stability.  It is meant to determine the distance that a person can move his/her center of gravity in the anterior direction before needing to take a compensatory step.

Safety first!

Have client stand next to a wall free of obstructions with the feet hip width apart.  The dominant arm should be closest to the wall.  Have client extend his/her arm to the front at 90 degree angle (to shoulder height).  The client should  not touch the wall or measurement device.  Client should reach forward without taking a step.  Measurement based on subtracting starting point from ending position.

Pass:  If reach is greater than 10 inches subject is unlikely to fall

Fail: if reach is < 10 inches  (note:  6-10 inches,  2x more likely to fall; < 6 inches,  4x more likely to fall)

Inches